Applications and Directions
Steven L. Waldbusser

In this issue: Exposing the Myths about Autotopology

Autotopology is often cited as the most requested network management feature in user surveys and by enthusiastic sales and marketing personnel. The dream of autotopology is that a management station can be installed and will automatically learn the configuration of the entire network and display it in a graphical form suitable for network management. This dream is often held by users of large networks daunted by the prospect of configuring hundreds or thousands of network devices by hand. While many aspects of this dream are possible, many others are impossible or are ill-advised.

Network management users need a graphical network map to display the network in a logical and useful manner. This requires a fair bit of information from the network. Autotopology can ease the burden of entering this volume of information into the Network Management System (NMS) by hand. Some network management architects wish to make their products simpler by using autotopology to make up for the lack of a database -- they simply re-discover the network configuration on every startup. Of course, the costs of this strategy are prohibitive for all but the smallest of internets.

There are three aspects to autotopology that should be explored in turn: topology discovery, node discovery, and drawing the map. There are difficulties in each of these areas, as well as ideas that may help make them more solvable.

Topology Discovery

When discovering the network topology, the network management system attempts to find all devices and networks that interconnect the internet, and to discover the interrelationships among all of these. This topology is made up of LANs, WANs, routers, bridges, hubs, repeaters, and so on. Often this discovery can be made by using SNMP to query for routing information from interconnection devices, from which a crude map may be constructed. Unfortunately, this type of discovery will not notice transparent devices such as bridges, hubs, and repeaters. A network manager typically needs to see these devices on a map to solve everyday problems. The standardization and implementation of MIBs for bridges and repeaters will provide access to forwarding and discovery information, and thus provide opportunities to add these devices to the map in the future. However, such opportunities will always be hindered by the inherent transparency of these devices.

Device Discovery

After discovering the network topology, a network management system needs to discover the devices that exist on the internet. One strategy that has been used is to send a broadcast SNMP packet that will compel every device to respond with its identity. This can be dangerous on mid-size or larger networks due to the storm of replies that will choke the network. There is also no guarantee that all devices have an SNMP agent or that the agent will respond. In any event, this mechanism must be used with care.

A second method of device discovery is to send an ICMP ECHO packet to every possible address on the internet, in sequence. This is referred to as the Monte Carlo method of network management (a term coined by Chuck Davin). This method consumes a lot of time and network resources. On a Class A IP network with 4 million possible addresses this would be impossible, but even on a Class B IP network it will generate at least 65,000 packets and take a lot of time (though it's probably the 65,000 ARP broadcast packets that will kill your network). This mechanism should be used with care as well.

A third method of device discovery is to use the passive monitoring capabilities of a Remote Network Monitoring device to discover devices from the source addresses of the packets they send. The Remote Network Monitoring (RMON) MIB defines objects that allow SNMP retrieval of a list of link-level addresses discovered by an RMON probe. The cost of dedicated remote monitoring systems could be a barrier to some users, but as routers, hubs, and other devices implement the RMON MIB, the cost can drop dramatically. The Internet Engineering Task Force (IETF) has chosen RMON as the preferred mechanism for device discovery, and will shortly embark on an effort within the RMON Working Group to add the objects for discovery of the addresses of network protocols such as IP.

To be effective, it is almost always necessary to store the discovered devices and topology in a database. It is also helpful for the NMS to automatically use SNMP to discover configuration information from devices and to store it in the database. This information can be invaluable in tracking down the source of a network failure, at which time it is probably impossible to query the devices directly. A good system will also have facilities for updating its database when it detects additions, replacements, or changes of network devices.

Map Drawing

The next step in autotopology is to draw a graphical map of the network, given the topology and devices that have been discovered. What is likely to result from this operation is a map containing hundreds or thousands of tiny icons without any recognizable structure. The reason that there is no recognizable structure is that the topological information is not sufficient to draw a network as one would view it logically. The NMS user typically views the network in geographical or administrative terms (or both), while the NMS can only discover the connectivity of the network. For example, the NMS doesn't have enough information to know that the LANs in the administrative building should be laid out side-by-side. It is equally likely to scatter them across the map. The user's aid must be enlisted to layout the map in a logical manner. With a well-designed network management station, this can be a simple matter of dragging icons and networks to get them placed correctly.

The Other Shoe Drops

After handling the layout problem, the biggest problem facing the NMS user is that the NMS doesn't have any concept of which devices are interesting to the user, so it either displays all of them or none of them. In fact, both cases are often nearly useless. It is very hard for the management station to understand what will be interesting to the user, so the user needs to configure this aspect of the system. The management station can help by choosing interesting nodes based on predefined criteria and placing these on the map. For example, the management station may choose to select all nodes with any of the following MIB-II characteristics:

- ipForwarding = gateway(1)

- ifNumber > 2

- sysObjectID identifies it as bridge, router, server, etc.

- ipOutRequests/sec > 100

Similar heuristics can be applied to other MIBs. In fact the presence or absence of the particular management objects that make up a MIB might signal that a device is interesting.

Clearly there are several reasons why autotopology cannot be truly automated. Automated systems usually can't get enough topology detail; they can't lay out the map in a logical fashion; and, they can't automatically decide which devices are important enough to deserve an icon on a crowded display. There are some ideas and algorithms which, if used, can make the system more usable and efficient. However, it is important to note that these systems will always require help from the network management user and will never completely satisfy the autotopology dream.